Thanxxx to all whose source codes I have used to compile this program – Tomas Vlad, Sa1vador, Pio, it would have been totally impossible to do that without having these codes.

To understand the material given further one needs to possess some definite knowledge of how to work with the computer and receiver using the JTAG interface. You will find that in Necessary Info.doc

# Control elements description

## Hardware config section

- Jtag Delay button – enables manipulation of data flow speed from computer to ТАP (test access port) of the receiver,  Check  button is next to Jtag Delay which enables you to automatically test if the JTAG functions OK. Pressing this button starts the transmission of sequence 0101010101 into TAP bypass register  , and immediately after it’s read from this register back. Two tests are conducted , a short one (64 bytes) and a long one (64 KB). You will be able to see the test’s results in an automatically opened window. If all is OK (the last string must show the  IDcode of your CPU), then remember Jtag Delay tracker’s position , this being the maximum speed that the computer is able to send data to the receiver without any mistakes. Do not though flatter yourself that everything is OK after this has been completed. In Tomas Vlad’s source code that I used as well, some delays during the data transmitting/receiving  through DCU have been foreseen  , and these delays depend upon the computer’s processor frequency clock. Thus, the speed can be additionally decreased a little bit to be just sure (i.е. increasing the delay value), because the test runs through bypass register without touching DCU. I have conveyed my tests setting TAP Clock frequency = 8 MHz. During the next program launch you may skip this check using the already known delay value (this value is saved in settings.ini file when you are exiting from program).

- Trap routine address – the address  under which the trap handler program will be written. To successfully write the flashes you will need to have at least  4,5 кbytes of memory that no other parallel process is using. This will be the allocation space for the very trap-handler, its variables (workspace), and for data buffer amounting 4 кbytes. The default value here was set up to 80000140, this is the start-up address of the CPU’s on-chip memory available for user. In some cases it’s possible to use the given address (well if you are lucky enough))), but sometimes the handler’s loading under this address leads to erroneous flashes’ writing. If you know the RAM and VideoRAM start-up addresses you can use them in RAM start address, Video RAM start address,  then press  Find Free Mem and wait until the program reads the memory and finds some free space (i.е. an uninterrupted memory area not less than 5 кbytes all filled with 1111or 0000; what is the search target  – 0 or 1, - depends on Free Memory Condition setup.  This process is rather a long one, in case you have no wish to wait anymore go to Flash Programming and press  Abort  button. After the memory check-up you will see the list of all free space memory areas found by the program in Trap routine address dropdown list.

- Trap routine parameters section

  - Trapped process Priority, Group of Traps-  these buttons allow to choose the priority and groups of traps which will be set up after pressing the button Install Traps. When you try to revive the tuner choose option High Priority Break Trap, other options are included here only for experimenting purposes. Additional Break Trap  parameters can be found in Miscellaneous, and they allow to check the processor’s behavior with different DCU Control Register values. Do please mark Trap on interrupt when you try to revive the tuner, it’s absolutely necessary to do this step.

  - Check Existing – you’ll be shown all the information about the existing configuration of traps (if all are 00000000 then either your JTAG interface does not function at all or TAP (test access port) hanged, so you will have to reboot the receiver. If this did not help out either, try increasing Jtag Delay value. In case even this did not help then you have a real problem  –  all by now tested receivers caused no obstacles when trying to read this area.

  - Read Int.Rom – (the  absolete button,  no more useful now, I’m going to remove it in the future release) your receiver’s loader will be read beginning from address 7FFF 0000, in case of  ВСТ-1530 this is a built-in RОМ, others can use it as senior flash addresses. The read file will be saved as  intROM.bin in the same directory you have copied the given program.

  - Load Trap Routine – upon pressing this button there will appear a “choose-a-file” dialog, the file containing the trap-handler. Different receivers need different algorithm to program their own Flash-memory chips. This is due to a physical connection of these chips to address and data buses. After the file needed has been chosen it will be loaded under the address provided in Trap routine address field.

  - Install Traps – this button will allow to load the parameters provided in Trap routine address under addresses of traps configuration  (addresses 80000040..8000013С).

  - Take Trap. The needed instruction defined by the traps configuration will be written into DCU. Pay attention to the info appearing in the log-window – if you see  DCU_CONTROL 00060000 or Trap workspace register OP_TEMP : ХХХХХХХХХ, where ХХХ is not equal 00000000, then this means ALL ОК, the trap has been captured and the trap-routine runs.

## Flash Programming

All following actions on this page  (excluding reading the flash content into a file) will be possible only after the trap handler has been successfully run !!! In case the trap routine did not install and doesn’t run, all erasing, writing, write protection check and IDCODE reading actions

WILL NOT WORK!!!

- Flash start address – flash start address, it may be entered manually or chosen from the provided list. After button Check was pressed, a chip’s IDCODE will be read. The proper value at this field absolutely necessary to correctly program or erase flashes.

- Sector address – full address of the sector which needs to be erased or checked on write-protection, e.g.  zero sector of the flash having the base address  7FE0 0000 - 7FE0 0000, the first sector  having 7FE1 0000 address, the second having 7FE2 0000 respectively, and so on. More detailed information on sectors’ sizes and their offsets you will find in manufacturer’s datasheets for YOUR type of chip. This field you will also need to erase sectors and to check whether they are write-protected. To erase the sector press Clear Sector, to check it - Block Protect Status (in log-window you will see a string “Block  ХХХХХХХХ  protection status = №,” ХХХ meaning the block address, № its status, if  №=1 – block is WP (Write-Protected), if 0 – WP is OFF). Pressing Clear Chip will perform erasing of the entire chip. This operation usually takes some time, typically 30 sec for 2Мbytes, but it can last up to 256 sec, as shown in the datasheets . Mind that protected sectors won’t be erased !!!  To erase them special measures should be undertaken!

- From address – the field necessary during reading and loading the memory content into a file. Writing can be performed ONLY INTO the flash, reading though is available from any addresses. Pressing the Read to File, Write from File buttons will open a standard dialog proposing you to choose a file. Upon reading the memory content into already existing file the content of this file will be previously erased .

- Data Length – well I presume this item is simple, denoting the data volume that are going to be written into a flash, or  the data volume which will be read from flash into a file. The field is necessary upon reading operations.

  - If this field contains  0, then during the flash writing the data volume will be defined automatically, equaling the file size minus File Offset value.

  - If some definite value (not zero) is provided in this field the flash will be rewritten not by a whole file but only by its part  .

- File Offset – sometimes you want it only to write a part of the definite file into a flash and not the whole file. Say we have a 2 Mb Firmware file, but I wish to write only its last 64 Kbytes into the flash. To do this I will have to enter 1F0000 (0х200000 –0х10000) value into Offset field , leaving Data Length  at  0 or entering 10000 there. After the button is pressed the flash will be rewritten with the data of the last 64 Kbytes of the chosen file.

- Verify after prg – if you mark this checkbox  the flash’s part that was rewritten will be verified with the source file.

- Compare with File – this allows to compare a part of flash equaling Data Length,  starting from the address shown in From address field, with a part of a binary file, the reading of the file will start from provided File Offset. If  Data Length=0, then the compared data volume will equal:

File Length minus File Offset.

Attention !!! In case the operation does not finish for rather a long time, and the progress bar below is not moving but you can see the dots running in the log window, that means the operation hanged and you’ve got to press the  «Abort !» button.  You will be prompted whether to  'Wait for normal termination ?'. If you click YES, an attempt to normally complete the operation will be undertaken, but if the operation still does not finish you will have to click NO and the operation will be forcibly terminated. Do please RESET the receiver after these procedures.

## DCU registers

- DCU monitor – shows what’s going on with CPU

  - Start  - DCU monitor ON/OFF. Attention !!! After the Start button was pressed ALL the rest buttons (but for a green one next field Control) would be BLOCKED ! Do not forget to unblock the buttons by pressing  Stop !

  - Polling interval – changes the DCU registers polling interval intensity.

  - Green button near the Control  field pops up a dialog of value changes of  DCU Control register. New value will be sent to register only in case the DCU monitor is ON.

- DCU Control - Batch Command Here a packet of commands changing the value of Control register. Values from the list will be sent to Control register in series of 50 millisecs after the Run Batch was pressed.  The idea of this manipulation is to ease the manipulation of bits of  Control register, used in well-known Polish recipe. It can be used for a variety of experiments.

  - Add Record – adds new value to the list of commands

  - Edit Record – allows editing of the highlited command.

  - Delete Record – deletes the highlited command .

- Manual Commands – you can manually read/write 4-bytes words from memory here. Reading is allowed from any accessible memory address, writing  is also allowed anywhere except flash chips.

  - Read. Reading of 4 bytes from address shown in Addr field to Data field.

  - Write  - writing of 4 bytes, entered in Data field,  under address shown in Addr field.

## Miscellaneous

Different useful “gadgets” that I was not able to include anywhere in my program have been grouped here.

- Break conditions – this switch denotes what values will be written into control registers of DCU on pressing Take Trap. Only Trap on interrupt used to function properly in most of my tries and cases, but you can try to use other values as well.

- RAM Writing Verification-  this checks (verifies) whether the RAM writing was successful. Do not use THIS option to write the flashes !!! You will fail anyway 

  - Verify RAM Load – allows to load under address in field RAM address ANY binary file. After the file has been loaded the program will compare the memory content with the source file and displays the results of the test.

  - Verify Trap Handler – after the trap handler was loaded in RAM, you can check at any point of time if it’s not been effaced by some other process. When you press this button the reading of the data from the address under which you have loaded the trap routine will be read ( Trap routine address ), the data obtained will be compared with the content of the file that was loaded in RAM after hitting the Load Trap Routine button, e.g.  fl4_trap.bin (that’s the trap handler itself). The check results will be displayed in the window.

Necessarily press this button after taking a trap !!! It provides more stable flow of the trap handler.

3.  EMI and Subsys Channels initialization.  In order CPU could correctly function with external memory, you need to configure the External Memory Interface (EMI) the right way. This is usually done by the loader, but if it was killed (erased or something), you will need to do this manually.

Pressing the Init from file button  will pop up the dialog window enabling you to choose the receiver type (it’s needed because different receivers have different initialization algorithms ). So you will have to simply choose YOUR decoder type and EMI will be initialized. Some loaders (e.g. like one of PACE), do not initialize DMA channels (i.e. the channels of direct memory access) of subsystems set up as registers under addresses 80000000..8000003С. To perform this Clear Traps button should be hit, and EMI will not be initialized, but values 80000000 will be written under addresses from 80000000 till   8000013С, as that’s done by Kenwoods’ loader.

Well this is generally it . Any questions, comments, suggestions and supplements are gladly welcome to be mailed to :  ivanov_dv@izvestia.ru.

English translation by Yaro (yaro@unet.net.ua)

CopyLeft 09/25/2002.

All comments in case of mistakes (if any) please send to the above shown address.